Workflow of the toolpath design platform

Step 1. Launch the application

Open the MEW toolpath design platform.

Step 2. Define the overall scaffold architecture

The overall scaffold architecture can be generated through one of the following three routes, depending on the design objective.

Route A: Select a pattern from the built-in library

- Choose a basic pattern from the built-in pattern library.

- Enter the required geometric parameters in the pop-up parameter input window.

- Preview the generated scaffold pattern in the display panel.

- Adjust the parameters, if necessary, until the desired pattern is obtained.

Route B: Import or create user-defined point-set patterns

- Prepare a user-defined pattern in TXT format, represented as a 2D point set.

- The starting point of the point set should be located at the coordinate origin (0,0), and the ending point should lie on the x-axis with a y-coordinate of 0.

- Alternatively, the point set can be created directly within the canvas by freely drawing the desired basic pattern, followed by optional editing operations such as vertex modification, rotation, or translation.

- Click “Save” to export the designed pattern as a point file. If required, repeat this procedure to generate two point files.

- Click “Load File” and import the prepared point-set file(s).

- Enter the corresponding parameters in the pop-up parameter input window.

- Preview the generated scaffold pattern in the display panel and adjust the parameters if necessary.

Route C: Import external images through the Eulerian-circuit module

- Prepare an externally defined pattern image in JPG or PNG format.

- Click “Load File” and import the image into the Eulerian-circuit generation module.

- The imported image is converted into a one-pixel-wide skeleton and then abstracted into a mathematical graph, in which intersections and endpoints are identified as vertices and the connecting segments are treated as edges.

- The Eulerian-circuit generation algorithm is then applied to transform the graph into a continuous printable toolpath.

- Preview the generated continuous path in the display panel and adjust the relevant parameters if necessary.

Step 3. Define the local structure (optional)

If local structural variation is required, additional transformations can be introduced after the overall scaffold architecture has been generated through Route A or Route B.

- Click “Region Selection” and use the mouse to define the target region within the generated scaffold pattern.

- Select a deformation mode from the “Transformation” list.

- Enter the required transformation parameters in the pop-up parameter input window.

- Preview the transformed pattern in the display panel and further adjust the parameters if necessary.

Step 4. Apply toolpath correction

- Click “Toolpath Correction” and enter the relevant process parameters in the pop-up parameter input window, including the jet lag length and printing speed.

- Based on these inputs, the platform corrects the originally designed toolpath and generates a new point set for printing.

- Preview the corrected toolpath and further adjust the parameters if necessary.

- The corrected point set is then used as the final printing path to improve fabrication accuracy in MEW.

Step 5. Close the application

Close the application after the design and export procedures are completed.